Active Control of Wind Turbine Aerodynamic Noise Using FXLMS Algorithm

Gloria Adewumi1*and Freddie Inambao2

1,2Green Energy Solution Research Group,

Discipline of Mechanical Engineering,

University of KwaZulu-Natal, Howard College,

Durban, Republic of South Africa.

*Corresponding email: 213574188@stu.ukzn.ac.za; adewumigloria@gmail.com

ABSTRACT

Active Noise Control (ANC) is a technique known to produce high level of attenuation in the low frequency range. This paper presents an initial study performed to investigate the possibilities of applying ANC on a wind turbine blade. Investigations were carried out to determine where the ANC system will be installed together with microphones placed strategically and close enough to determine aerodynamic noise. When installed, they were used to measure aerodynamic noise generated which in turn was compared with the noise produced when ANC is installed. The result of this investigation was then implemented to reduce noise in wind turbine blades. While carrying out this study, many factors were considered such as: turbulence, directivity, types of noise generated by the blades and the type of control system that was to be used. Furthermore, a Feed-forward ANC system based on the FXLMS algorithm was applied. Computer simulation, using MATLAB shows how the source noise and the anti-noise together with how they cancelled out are presented.

KEYWORDS: Wind Turbine, Aerodynamics, Noise, FXLMS Algorithm, ANC, Directivity.

- INTRODUCTION

Beginning from the nineteenth century onwards, Wind turbines became popular and have gone on to be a source of mechanical power for the last thousand years [1]. The energy produced from Wind is one of the major sources of renewable energy. It is clean, with no emissions, nonpolluting and can be claimed to be the cheapest method of energy generation. However, noise is among the problems associated with Wind Turbines; they are not known to cause hearing loss, but they produce low frequency noise and vibration which may have adverse health effects on humans [2] and has become an important concern globally [3]. The occurrence of noise depends on the level of acoustic emission of the turbine and secondly on the distance between the turbine and nearest residences [4]. The noise from wind turbines are categorized into mechanical noise and aerodynamic noise. The objective of this study is to introduce an opposite phase that is generated and combined with the primary “anti-noise” wave through an appropriate array of secondary noise, developed using FXLMS algorithm to minimize residual error, which consequently leads to reduction of low frequency aerodynamic noise from wind turbines.

1.1	CLASSIFICATION OF NOISE IN A WIND TURBINE

Mechanical noise is associated with the rotation of the mechanical and electrical components and it produces tonal sounds which can be described as a hum or whine. This type of noise is developed from the relative motion of mechanical components in the nacelle and the wind turbine tower and can be transmitted and radiated by the hub [3]. Aerodynamic noise on the other hand is as a result of the flow of air around the blades. This is the major source of noise from the turbine and it generally increases with rotor speed and it produces a broadband noise which is usually described as “swishing” or “whooshing” sound. Aerodynamic noise is divided into three [3]:

- Inflow Turbulence- this is due to atmospheric turbulence which results in local force or local pressure fluctuations around the blade.

- Low frequency noise- this is concerned with the low frequency part of the sound spectrum which is generated when the blade while rotating encounters localized flow deficiencies due to the flow around a tower, wakes and shed from other blades and wind speed changes. This is also referred to as tip noise.

- Airfoil self-noise- this includes noise generated by flow of air along the surface of the airfoil. In general, there are six regions in which air foil self-noise are found on a blade as illustrated in Fig. 1.

Modern active noise control is generally achieved through the use of analog circuits or digital signal processing. Adaptive algorithms are designed to analyze the waveform of the background aural or non-aural noise, then based on the specific algorithm, generates a signal that will either phase shift or invert the polarity of the original signal. This inverted signal is then amplified and a transducer creates a sound wave directly proportional to the amplitude of the original waveform, creating destructive interference. This effectively reduces the volume of the perceivable noise.

Turbulent incoming flow

Boundary layer transition

Trailing edge flow

Wake

U

Possible leading edge separation

Tip vortex

Fig. 1 Sources of noise on a wind turbine blade [4].

1.2	Past work on ANC

It is a known fact that ANC is the preferred approach when it comes to reducing low frequency noise. It consists of a reference microphone, a controller and an error microphone as shown in Fig.3. No previous research has been conducted in the field of noise reduction on wind turbine blades using ANC; however extensive research has been carried out on noise reduction using passive methods [5], [3], [6], [7], [8], [9], and [10]. This method was able to reduce noise but had some disadvantages [7], [4], [10] but on the other hand, the technique of using ANC for attenuation of fan noise has proven that it is a very efficient method [11], [12],[13],[14], [15], [16],[17],[18]. The reduction of tonal noise from large-diameter spray-dryer exhaust stacks was carried out by [19], using sufficient in-duct control sources and error sensors. Investigation for turbofan engine noise using ANC was carried out by [20] where a special fan rig called “Advanced noise control fan” was built and used in a wide range of experiment. Unlike noise propagated through a duct or enclosed region, Wind turbines operate openly and the noise it emits is radiated in free field [21], and the arrangement for Active noise control of the noise is shown in Fig.2.

Unbaffled oscillating piston

x

Equivalent fan

Ʋ1s

Ʋ1

r

rs

a

zs

z

p(r,ʋ1)

ṝ1

Fig. 2 Active noise control arrangement for a free-field fan noise [14].

An important reason why active noise control is more important than passive noise control at low frequencies [13], [22] is that ANC uses a digital signal processing (DSP) system as the canceler, which can execute complicated mathematical operations with enough speed and precision in real time [23]. ANC systems have the advantage of being small, compact, environmentally adjustable, and they are cost effective [24]. The success of an ANC system depends mainly on fulfilling two main criteria: the anti-noise waveform must closely match the shape and frequency of the noise waveform and the wave must be precisely 180o out of phase with respect to the original noise waveform, when reached to the target area [25], [24], or else a second acoustic noise is generated. For a highly effective cancelling system, the source of noise must be nearly stationary in relation to the speaker emitting the anti-noise waveform; secondly, the noise source should be located very close to the ANC system and for the best result, the target noise should be dominantly radiating in one direction [24].

ANC systems can be broadly classified into Feed-forward and Feed-back control [26], or a combination of hybrid ANC [27], [28].  Depending on the noise signal and environmental characteristics, they both have their advantage and disadvantages [29], [30].

An ANC has to be adaptive due to environmental changes, system component degradation and noise source alteration [30], and are generally designed on the basis of a mathematical description and its linearized model [30]. Acoustic delay is also a factor that must be sufficiently dealt with in a noise cancelling system [24].

In this paper, a single-channel feed-forward ANC system with one reference microphone and one error microphone will be used.

ANC’s have also been used in high-speed elevator in which various transfer paths of noise are transmitted from motor and rope to cabin interior. A noise reduction of 8dB was achieved in the cabin at ear level [31]. It was also effective in attenuating impulsive noise which occurs in channels which suffer from switching, manual interruptions, ignition noise and lightning, for example Digital Subscriber Line systems (DSL) and digital TV [32], and the focus of the author was on removing additive impulsive noise from a lowpass signal [33].

Unknown plant P(z)

Adaptive filter W(t)

LMS

S(t)

Σ

b(n)

y(n)

y’(n)

e(n)

a(n)

.

Fig. 3 Simplified block diagram of an ANC system (Lopez-Caudana et al, 2008)

- MATERIALS/METHODS

Acoustic noise measurements were taken according to [34], and implemented in the simple model presented in Fig.4. The primary source emits a wave P(t). This wave is detected by a sensor placed at a distance rt relative to the primary source and a distance ru relative to the secondary source and fed to the Adaptive controller C. After the detected signal has been adjusted in phase and amplitude, it is emitted by the secondary source to be superimposed on the unwanted noise. The result of this superposition is then observed at an observation point located at a distance rv relative to the primary source and a distance rw relative to the secondary source.

From the block diagram in Fig.5 [35]:

T(s)	Transfer function of path rt

U(s)	Transfer function of path ru

V(s)	Transfer function of path rv

W(s)	Transfer function of path rw

M(s)	Transfer function of detector

C(s)	Transfer function of the controller

N(s)	Transfer function of necessary electronics

L(s)	Transfer function of the secondary source

P(s)	Primary source output

S(s)	Secondary source output

The detector output D(s) and secondary source output S(s) are:

Ds=TsPs+UsSs        									(1)

Fig. 4 Schematic diagram of a Feed-forward control system [35].

Fig. 5 Block diagram of Feed-forward control system [35].

And

Ss=MsCsNsLsDs 									(2)

Substituting for D(s) from Eq. (1) into S(s) in Eq. (2):

S(s)=M(s)C(s)N(s)L(s)[T(s)P(s)+U(s)S(s)							(3)

And the transfer function A(s) between the secondary source S(s) and primary source P(s) output is given as:

As= S(s)P(s)= MsCsNsT(s)1-MsCsNsLsU(s) 								(4)

To cancel noise at the observer position,

P0s= -S0 (s)											(5)

From the Fig.5:

P0s=VsPs										(6)

S0 s=WsSs

Ss=AsP(s)											(7)

Substituting for P0s and S0 s from Eq. (6) into Eq. (5) using Eq. (7) gives:

Vs=-WsA(s)										(8)

The equation given above is the relation under which full cancellation of noise is achieved at the observation point. Solving for Cs yields:

Cs=V(s)MsNsL(s)∆(s)										(9)

Where:

∆s=UsVs-T(s)W(s)									(10)

A feed-forward control system is an arrangement in which the basis for the disturbance sensed can be used as a reference for control signal generation [36], thereby removing the non-zero restriction on the error signal [25]. Many on the electronic systems used in feed-forward control systems derive control inputs via modified adaptive signal processing/architecture combinations. Adaptive signal processing is a field born out of the requirements of modern telecommunications systems, where there is a need to filter a signal, so it can be extracted from contaminating noise. In this context, a digital signal processor will be implemented to filter the background surrounding noise from the wind turbine noise. Feed-forward control system as comprised of two parts, the ‘physical’ control system (actuators and sensors) and the ‘electronic’ control system [25], and it was further explained that the feed-forward input carries out the modification of the zeroes of the system in which it is being implemented. The equations of motion for a case in which the control input is introduced to cancel out a primary noise or disturbance is given as:

ẋt=Axt+but+w(t) ẋ(t)=Ax(t)+bu(t)+w(t) 									(11)

yt=cx(t)

Where w(t) is an (nx1) primary disturbance vector and it is equal to m (which is the transfer function between the input sinusoid r(t) and w(t)) given as:

wt=mr(t) 											(12)

If the feed-forward control input’s objective is to cancel the primary disturbance at the error sensor, it can be shown that:

cmrt+Nbrt=0 										(13)

Substituting for Eq. (13) into Eq. (12), and representing the result in frequency domain, output between the output y and reference signal r is:

y(s)r(s)=csI-A-1Nb+mr(s) 									(14)

From Eq. 13, it follows that:

y(s)r(s)=0												(15)

This shows that the controller for a feed-forward system places zero transmission at the reference signal frequency.

The feed-forward ANC structure is able to handle both narrowband noise and broadband noise, but the hybrid structure presented by [28], which consists of a feed-forward structure, was used to estimate the noise path and a feedback structure used to cancel the feed-back acoustic noise

2.1	STUDY DESIGN

In the course of carrying out this research, the need to determine the tone levels, masking noise levels, tonality and declaration of sound power levels of the turbine was required. Also, experiments were carried out from which data for conclusion will be derived.

Determination of the masking noise levels Lpn,iT

The 12 sound pressure levels of the masking noise Lpn,I are defined as follows:

Lpn= Lpn,avg +10Lg critical bandwidtheffective noise bandwidth	     		(16)

Where: Lpn,avg is the energy average of the spectral lines identified as masking.

Determination of the tonality ∆Ltn

The tonality ∆Ltn,i is the difference between the sound pressure level Lpt,i and the level Lpn,i. the 12 ∆Ltn,i are then energy averaged to one ∆Ltn

Declaration of Sound Power Level, Sound Pressure Level and Tonality Levels of wind turbines,

The sound power level of a source, Lw , in units of decibels (dB) is given by:

Lw  = 10 log10(PP0)                                                                                                				(17)

With P equal to the sound power of the source and P0  a reference sound power (usually 10-12 Watts)

The declared sound power levels for a wind turbine can be determined from n measurements results {Li}=1,…..n obtained by performing one measurement at each of n individual turbines of the same type. The n measurements results in a mean value Lw and a standard deviation s is defined as follows:

Lw= i=1nLin                                                                                                       				(18)

S= 1/(n-1)i=1nLi-Lw2                                                                   				(19)

The standard deviation of production σp can be estimated from:

(s2-σ2) ≤ σp≤s                                     								(20)

An estimate of the standard deviation of reproducibility σR is 0.9dB (given in Annex D of [34]).

The standard deviation σ used for the declaration (including the standard deviation σR and σp) from the n existing measurements and the standard deviation σR and σpof verification measurement is then determined by:

σ=1nσR2+σP2+σR2+σP2                                                                       				(21)

= 1+nnσR2+σP2                                                                                                                     		(22)

With σR= 0.9dB and σp=s

The declared sound power level is calculated from:

Lwd= Lw+ k= Lw+1.645 σ                                                                                    				(23)

The sound power level shall be declared by dual power noise-emission values reporting both Lw and k. k represents a certain confidence level and k= 1.645. σ Represents a probability of 5% that a sound power level measurement level result made according to [34] performed at a turbine of the batch exceeds the declared value.

Sound level meters are the instruments used to measure Sound Pressure Levels. They are recorded on a meter and make use of a microphone that converts pressure variations into a voltage signal [3]. The sound pressure level (SPL) of a noise, Lp , in units of decibels (dB), is given by:

Lp =  20 log10(pp0)                                                                                                   				(24)

With p equal to the effective (or root mean square, rms) sound pressure and p0 a reference rms sound pressure (usually 20 x 10-5 )

Broadband energy is created by the interaction of turbulence with the leading and trailing edges. Turbulence leading-edge interaction noise is dominated by the spectrum of the inflow turbulence in the atmospheric boundary layer. The peak energy for this type of noise is contained at a frequency. The frequency of the peak energy is given as [37]:

fpeak = StVtiph-0.7R   											(25)

With:

St	16.6

h	hub height  (m)

Vtip	rotor tip speed (rpm)

R	blade radius (m)

And the first order Blade Tower Interaction (BTI) interaction noise source strength is given as [38]:

LDTVtip qcl= 2παDTVtip											(26)

Where:

L	Time derivative of lift

DT	Tower diameter (m)

q	Dynamic pressure of the flow approaching the blade tip (N/m2)

c	Blade chord

L	Span wise region of the blade

α 	Time derivative of the blade angle of attack

- RESULTS AND DISCUSSION

The performance of FXLMS algorithm in reducing wind turbine aerodynamic noise is illustrated in this section. ANC scenarios are simulated in a MATLAB environment. The impulse responses of the primary, secondary and secondary path estimates were modelled (Fig. 6, 7, 8 and 9).

For this simulation, the noise was sampled at 22050Hz for 2 sec and it combines the noise signal, the FXLMS filter and the primary path filter. An FIR filter was used for the primary propagation path, and it was band limited from 200 to 800Hz and for the secondary path, it was 160 to 2000Hz. The simulation was in three stages: importation of the noise signal into MATLAB workspace, processing the noise through the primary filter and finally processing the output through FXLMS filter.

Fig.6 Primary path impulse response			Rref=H+D2Fig. 7 True secondary path impulse response

Fig. 8 Secondary path impulse response			       Fig.9 Secondary path impulse response estimation

Even though a wide range of algorithms can be used to design the secondary propagation path estimate, the Normalized LMS algorithm was used because of its robustness and simplicity. Plots of output and error signals are shown in Fig. 10.

Fig.10 Secondary identification using NLMS algorithm               Fig. 11 Power Spectral Density measurements

The Welch Power Spectral Density (PSD) estimate using spectral estimation of the signal was computed. Then the main frequency was found by looking at the spectral density and locating the frequency at which the PSD reaches its maximum, this was found to be at 0.08613 KHz (86.13Hz).  This is therefore the “true frequency”. This continuous wavelet transform of the signal was computed and the spectral information is presented in Fig. 11.

Fig. 12 Original and attenuated noise

Fig 13a- Magnitude and 13b- phase response of the original signal       Fig 14a-Magnitude and 14b-phase of the attenuated signal

For the MATLAB implementation the frequency domain plots above represents information about the turbine aerodynamic noise magnitude and phase at each frequency. The magnitude response shows the strength of the frequency components while the phase shows how all the frequency components aligns in time. From the fig 13a, it can be seen that the highest magnitude of noise is found between 0 to 200 Hz. This indicates that the noise has low frequency components. Fig. 13 and 14 shows the magnitude and phase responses for both the original and attenuated signals.

- CONCLUSIONS

- An active noise control system is proposed for reducing the noise level from wind turbine blades using the FXLMS adaptive filter. The input to the system is noise signal which was measured according to IEC 61400-II standard. NLMS algorithm was used for updating filter coefficients. The implementation in MATLAB environment has been described. FXLMS algorithm was able to minimize residual error. The results of the simulation shows that noise was reduced by about 29dB. This approach can be used to attenuate wind turbine aerodynamic noise emmisions and noise in fans radiating in a free field.

- ACKNOWLEDGMENT

This work was carried out by the Green Energy Solution (GES) of the University of KwaZulu-Natal, Howard College, Durban.

- REFERENCES

[1]   D.G. Shepherd, Historical Development of the Windmill, National Aeronautics and Space Administration, Lewis Research Centre, Cornell University, Ithaca,

New York, 1990.

[2]   H. Møller, C.S. Pederson, Low Frequency Noise from Large Wind Turbines, Journal of Acoustical Society of America 129(2011) 3727-3744.

[3]   L.A. Rogers, F.J Manwell, Wind Turbine Noise Issues, Renewable Energy Research Laboratory, 2004.

[4]   O. Jianu, A.M. Rosen, G. Naterer, Noise Pollution Prevention in Wind Turbines: Status and Recent Advances, Journal of Sustainability 4(2012) 1104-1117.

[5]   T. Göçmen, B. Özerdem, Airfoil Optimization for Noise Emission problem and Aerodynamic performance criterion on Small Scale Wind Turbines, Journal of

Energy 46(2012) 62-71

[6]   J.G Schepers, A. Curvers, S, Oerlemans, K. Braun, T. Lutz, A.Herrig, W. Wuerz, B.Mendez-Lopez, SIRROCO: Silent Rotors by Acoustic Optimization,

Proceedings at the Second International Meeting on Wind Turbine Noise, (2007).

[7]   S. Oerlemans, M. Fisher, T. Maeder, K. Kogler, Reduction of Wind Turbine Noise using Optimized Airfoils and Trailing Edge Serrations, AIAA Journal

47(2009) 3-4.

[8]   G. Leloudas, W.J Zhu, J.N Sorenson, W.Z Shen, S. Hjort, Prediction and Reduction of Noise from a 2.3MW Wind Turbine.Journal of physics: conference

series  75(2007) 1-10.

[9]   M.S Howe, A Review of the Theory of Trailing Edge noise, Journal of Sound and Vibration 61 (1978) 437-465.

[10] T.Geyer, E. Sarradj, C. Fritzçche, Measurement of the Noise Generation at the Trailing Edge of Porous Airfoils, Experiments in Fluids 48 (2010) 391-308.

[11] G.C Lauchle, J.R Mac Gillivray, D.C Swanson, Active Control of Axial-Flow Fan Noise, Journal of the acoustical society of America 101 (1997) 341-349.

[12] Y.J Wong, R. Paurobally, J. Pan, Hybrid Active and Passive Control of Fan Noise. Applied Acoustics 64(2003) 885-901.

[13] M.S Murthy, M.G.A Elnourani, Active Noise Control of a Radial Fan. Thesis for master’s degree. Blekinge Institute of Technology, 2008.

[14] A.Gérard, A. Berry, P. Masson, Control of Tonal Noise from Subsonic Axial Fan. Part 2: Active Control Simulations and Experiments in Free Field,  Journal

of sound and Vibration 288(2005) 1077-1104.

[15] H.A Coudourier-Maruri, F. Orduña-Bustamante, Active Control of Periodic Fan Noise in Laptops: Spectral Width Requirements in a Delayed Buffer

Implementation, Journal of Applied Research and Technology 7 (2009) 124-135.

[16] J. Wang, L.Huang, Active Control of Drag Noise from a Small Axial Flow Fan, Journal of Acoustical Society of America 120(2009) 192-203.

[17] B. Dragan, F, Taraboanta, Active Noise Control of Axial Fans, The Annals of University “Dunarea de Jos” of Galati (2002) 76-79.

[18] G.E. Piper, J.M Watkins, O.G. Thorp, Active Control of Axial-Flow Fan Noise Using Magnetic Bearings, Journal of Vibration and Control 11(2005) 1221-

1232

[19] X. Li, X. Qui, D.L.L Leclercq, A.C Zander, C.H Hansen, Implementation of Active Noise Control in a Multi-Modal Spray Dryer Exhaust Stack, 67(2006) 28-

48.

[20] D.L Huff, Noise Reduction Technologies for Turbofan Engines, National Aeronautics and Space Administration  (2007).

[21] S. Lee, S. Lee, S. Lee, Numerical Modelling of Wind of Wind Turbine Aerodynamic Noise in the time domain, Journal of Acoustical Society of America

133(2013) 94-100.

[22] S. Kottayi, N.K Narayanan, ANC System for Noisy Speech, Signal and image processing: An International Journal (SIPJ) 3( 2012) 93-101.

[23] R. Zhu, Active Control of Impulsive Noise. Thesis for master’s degree, University of Minnesota Duluth, 2009.

[24] R. Hashemian, Design of an Active Noise Control System using Combinations of DSP and FPGAs, PLD conference proceedings (2006) 21-26.

[25] C. Hansen, S.D Snyder, Active Control of Noise and Vibration, E and FN, London UK. 1997

[26] N.V George, G. Panda, Advances in in Active Noise Control: A Survey, with Emphasis on Recent Non-linear Techniques, Journal of signal processing

93(2013) 363-377.

[27] S.M Kuo, I. Panahi, K.M Chung, T. Horner, M. Nadeski, J. Chyan, Design of Active Noise Control Systems with the TMS320 Family. Application report,

Texas Instruments 1996.

[28] E. Lopez-Caudana, P. Betancourt, E. Cruz, M. Nakano-Miyatake, H. Perez-Meana, An Active Noise Cancelling Algorithm with Secondary Path Modelling,

12th WSEAS International Conference on COMPUTERS 12(2008) 194-199.

[29] S.B Behera, D.P Das, N.K Rout, Non-Linear Feedback Active Noise Control for Broadband Chaotic Noise. Journal of applied soft computing 15(2013) 80-87.

[30] C. Chang, F. Luoh, Enhancement of Active Noise Control Using Neural based Filtered-X Algorithm.

Journal of sound and vibration 305(2007) 348-356.

[31] I.Yang, J. Jeong, U. Jeong, J. Kim, J. Oh, Improvement of Noise Reduction Performance for a High Speed Elevator using Modified Active Noise Control,

Journal of applied acoustics 79(2013) 58-68.

[32] S. Zahedpour, S. Feizi, A. Amini, M. Ferdosizadeh, F. Marvasti, Impulsive Noise Cancellation Based on Soft Decision and Recursion, IEEE Transactions on

instrumentation and measurement  58(2009) 2780-2790.

[33] X. Sun, S.M Kuo, G. Meng, Adaptive Algorithm for Active Control of Impulsive Noise, Journal of sound and vibration, 291(2006)  516-522.

[34] IEC 61400-11 ed 3.0. Wind Turbines-Part 11: Acoustic Noise Measurement Techniques, 2012.

[35] D.T. Swift-Hook, Wind Energy and the Environment, Peter Peregrinus Ltd. United Kingdom 1989.

[36] B. Pathak, P.P Hirave, FXLMS Algorithm for Feed-Forward Active Noise Cancellation, Proceedings of the International Conference on advances in

Computer (2012) 18-22.

[37] S. Wagner, R.Bareiss, G. Guidati, Wind Turbine Noise, Springer Verlag, 1996.

[38] C. Doolan, Wind Turbine Noise Mechanisms and some Concepts for its Control, Proceedings of ACOUSTICS 2011 17(2011) 1-7.